TWB Observation Report — 08 Dec 2013

| Observatory: | AJ4CO Observatory | Instrumentation: | TWB on TFD Array RCP |
| --- | --- | --- | --- |
| Location: | High Springs, Florida |  | Gage CS1220 Digitizer |
| Observer: | Dave Typinski |  | DPS on TFD Array |

| Observer On Duty (UTC): | 0350 | Body: | Jupiter |
| --- | --- | --- | --- |
| Observer Off Duty (UTC): | 0615 | Storm Type: | Io-A/C |

| Storm Activity (UTC) |  | CML-III | Io  | Off Axis | DE | JS  |  JE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Begin: | 0400 | 191° | 238° | 51° | +1.61° | –147° | 4.33 AU |
| End: | 0715 | 309° | 266° | 10° |  |  |  |

| Interval |  |  |  |  |  | Fhi | Flo |  |  |  | Drift |  | Rot Rate | TEC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # | (UTC) | L | S | N | Pol | MHz | MHz | CML | Io | S | MHz/sec | FB | #/MHz @ F | 1016/m2 |
| 1 | 0520–0600 | X |  |  | RCP | 17 | 26 | 250° | 252° |  |  |  |  | 17.1 |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Interval – Time during which something interesting was observed L, S, N – Indicates presence of L bursts, S bursts, and N events Pol – Dominant circular polarization observed Fhi, Flo – Frequency range of observed emission CML, Io – System III CML and Io phase during noted interval | ML – indicates presence of modulation lanes Drift – Observed modulation lane drift rate in MHz per minute FB – indicates presence of Faraday bands Rot Rate – Observed Faraday rotation rate in rotations per MHz at observing frequency F TEC – Ionospheric vertical total electron count during noted interval as reported by NGDC |
| --- | --- |

Observer Notes

The TFD array does not do well with off-axis angles > 30°. This is due to the way the beam changes shape with frequency and off-axis angle. First-look analysis of the high speed data evinced some weak S bursts mixed with L bursts ca. 0530.

| Data File(s): | AJ4CO-DPS-131208040000.sps plus 32 Gage data files |  |
| --- | --- | --- |

| Custom Interval Ephemerides |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Start Dates: | 12/8/13 |  |  |  | 12/8/13 |
|  | & End Times: | 04:00:00 |  |  |  | 07:15:00 |
|  |  | Interval Start |  | Interval Center |  | Interval End |
| Date |  | 08 Dec 2013 |  | 08 Dec 2013 |  | 08 Dec 2013 |
| UTC |  | 04:00:00 |  | 05:37:30 |  | 07:15:00 |
| JDE |  | 2456634.66746 |  | 2456634.73517 |  | 2456634.80288 |
| Local (UTC-4) |  | 00:00:00 |  | 01:37:30 |  | 03:15:00 |
| LMST |  | 03:37:49 |  | 05:15:35 |  | 06:53:21 |
|  |  | Jupiter |  | Jupiter |  | Jupiter |
| Alt ° |  | +39.499 |  | +60.662 |  | +79.983 |
| Az ° |  | +84.790 |  | +97.634 |  | +138.308 |
| HA hms |  | - 03:44:18 |  | - 02:06:30 |  | - 00:28:42 |
| Off Axis ° |  | +50.501 |  | +29.338 |  | +10.017 |
| RA hms |  | - 16:37:52.8673 |  | - 16:37:54.5371 |  | - 16:37:56.2098 |
| Dec ° |  | +22.1630688 |  | +22.1641406 |  | +22.1652138 |
| Rise UTC |  | 0049 (2049 L) |  | 0049 (2049 L) |  | 0049 (2049 L) |
| Trans UTC |  | 0743 (0343 L) |  | 0743 (0343 L) |  | 0743 (0343 L) |
| Set UTC |  | 1437 (1037 L) |  | 1437 (1037 L) |  | 1437 (1037 L) |
| De ° |  | +1.61176550 |  | +1.61176885 |  | +1.61177245 |
|  J-E AU |  | +4.32725188 |  | +4.32669846 |  | +4.32614622 |
| J-E ° |  | +26.83062 |  | +26.76751 |  | +26.70440 |
|  |  |  |  |  |  |  |
| Ds ° |  | +1.76367034 |  | +1.76341563 |  | +1.76316089 |
|  J-S AU |  | +5.183374472 |  | +5.183399477 |  | +5.183424481 |
| J-S ° |  | - 147.27193 |  | - 147.34724 |  | - 147.42255 |
| CML-III ° |  | +191.458 |  | +250.410 |  | +309.361 |
| Io SGC ° |  | +238.151 |  | +251.902 |  | +265.628 |
| Io  (III) ° |  |  |  |  |  |  |

Io-CML Phase Plane

RJP Sky View

US-TEC Map

| Kp index, 3-hour average: | 5 |
| --- | --- |

CONUS Lightning Activity

| Instrumentation: | DPS on XTFD 60 kHz IF 300 channels, 53 kHz spacing 17–33 MHz |
| --- | --- |

Calibration Scale

Overview Spectrogram (700 px)