TWB Observation Report — 15 Dec 2013

| Observatory: | AJ4CO Observatory | Instrumentation: | TWB on TFD Array RCP |
| --- | --- | --- | --- |
| Location: | High Springs, Florida |  | Gage CS1220 Digitizer |
| Observer: | Dave Typinski |  | DPS on TFD Array |

| Observer On Duty (UTC): | 0430 | Body: | Jupiter |
| --- | --- | --- | --- |
| Observer Off Duty (UTC): | 0745 | Storm Type: | Io-A |

| Storm Activity (UTC) |  | CML-III | Io  | Off Axis | DE | JS  |  JE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Begin: | 0500 | 202° | 232° | 15° | +1.61° | –155° | 4.28 AU |
| End: | 0715 | 284° | 251° | 3° |  |  |  |

| Interval |  |  |  |  |  | Fhi | Flo |  |  |  | Drift |  | Rot Rate | TEC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # | (UTC) | L | S | N | Pol | MHz | MHz | CML | Io | S | MHz/sec | FB | #/MHz @ F | 1016/m2 |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 13.8 |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Interval – Time during which something interesting was observed L, S, N – Indicates presence of L bursts, S bursts, and N events Pol – Dominant circular polarization observed Fhi, Flo – Frequency range of observed emission CML, Io – System III CML and Io phase during noted interval | ML – indicates presence of modulation lanes Drift – Observed modulation lane drift rate in MHz per minute FB – indicates presence of Faraday bands Rot Rate – Observed Faraday rotation rate in rotations per MHz at observing frequency F TEC – Ionospheric vertical total electron count during noted interval as reported by NGDC |
| --- | --- |

Observer Notes

L bursting. Beam steered-TFD array seems to possibly produce Faraday like banding in its output—check against zenith steered array for Io-A on 12/22/13.

| Data File(s): | AJ4CO-DPS-131215050000.sps plus 33 Gage data files |  |
| --- | --- | --- |

Io-CML Phase Plane

RJP Sky View

US-TEC Map

| Kp index, 3-hour average: | 1 |
| --- | --- |

CONUS Lightning Activity

| Instrumentation: | DPS on XTFD 60 kHz IF 300 channels, 53 kHz spacing 17–33 MHz |
| --- | --- |

Calibration Scale

Overview Spectrograms (700 px)